-----------------------------------------------------------------

The following is a news release from Master Bond Inc. You have received it because you are listed as an editor for your publication.

Attached to this email is a low resolution version of the photograph that is included in the press kit for this product.

A high resolution version of this image and files with the body text of this release in Word, HTML and text formats are available at https://www.masterbond.com/newsrelease/led415dc90.

-----------------------------------------------------------------

## FOR IMMEDIATE RELEASE

Dual Curable Adhesive Offers Rapid Fixturing with LED Light

Master Bond LED415DC90 is a one component adhesive system that cures rapidly upon exposure to a 405 nm wavelength LED light source without any oxygen inhibition. The rate of cure depends upon the intensity of the light source, the thickness of the adhesive layer and the distance from the light to the adhesive. This no-mix system requires a post cure with heat, and adheres well to both similar and dissimilar substrates.

“Conventional light curing adhesives do not allow light penetration beyond 1 mm.  LED415DC90’s key distinguishing factor is its ability to partially cure or fixture up to 6 mm deep or in cross sections under LED light at the right intensity,” says Rohit Ramnath, Senior Product Engineer. “That enables a unique practical feature of this product: fixturing of components which are opaque. The mechanism in which this occurs is by allowing the light to penetrate through the adhesive from the sides and not needing light to be exposed throughout the entire areas.  It must be noted that post curing with heat at about 95°C for 30-60 minutes is critical for enhancing strength and overall performance.”

LED415DC90 has a thixotropic index of 5.53 at room temperature. It has a good physical strength profile with a tensile strength of 5,500-6,500 psi, a tensile modulus of 450,000-550,000 psi, and lap shear strength for Aluminum to Aluminum of more than 1,000 psi. This rigid system cures with a hardness of around 85-90 Shore D and elongation of 1-3% at 75°F. Serviceable from -80°F to +350°F, it is electrically non-conductive with a volume resistivity greater than 1014 ohm-cm. It is available in EFD® syringes, ½ pint, pint and quart containers.

Master Bond Dual Curing Adhesives

Master Bond LED415DC90 is a single component adhesive that utilizes both LED light and heat for curing and is ideal for applications where ultra-fast fixturing is critical. It is thixotropic, high strength, and electrically insulative. Read more about Master Bond’s dual curing adhesives at https://www.masterbond.com/products/dual-curing-adhesive-systems or contact Tech Support. Phone: +1-201-343-8983 Fax: +1-201-343-2132 Email: technical@masterbond.com.

Note to Editors:

For a full product description, please visit: https://www.masterbond.com/tds/led415dc90

Check out new videos on our YouTube channel: http://www.youtube.com/user/MasterBondVideo

You can embed any of our videos on your website.

CONTACT

James Brenner, Marketing Manager

Email: jbrenner@masterbond.com

Tel: +1-201-343-8983

Fax: +1-201-343-2132

MASTER BOND INC.

154 Hobart Street

Hackensack, NJ 07601-3922

Web: www.masterbond.com

# # #