The following is a news release from Master Bond Inc. You have received it because you are listed as an editor for your publication.

Attached to this email is a low resolution version of the photograph that is included in the press kit for this product.

High resolution version of this image and files with the body text of this release in Word, HTML and text formats are available at http://www.masterbond.com/newsrelease/supreme-62-1.

-----------------------------------------------------------------

## FOR IMMEDIATE RELEASE

Toughened Two Part Epoxy Withstands Repeated Thermal Cycling

Master Bond Inc. has developed a solvent free, two component, toughened epoxy system called Supreme 62-1.  It is serviceable from -60°F to +450°F (-51°C to +232°C). Most noteworthy, Supreme 62-1 provides chemical resistance to a wide range of acids, bases, fuels and solvents even at elevated temperatures. It can be used as an adhesive/sealant for aerospace, electronic, optical and specialty OEM applications.

“Master Bond Supreme 62-1 has superior toughness, which makes it suitable for bonding substrates with different coefficients of thermal expansion and allows it to resist repeated thermal cycling”, says Rohit Ramnath, senior product engineer. “This formulation also exhibits a tensile strength of 8,000-9,000 psi and a tensile modulus of 450,000-500,000 psi. Based on its combination of thermal stability and high physical strength profile we can recommend Supreme 62-1 for many applications requiring structural bonding of dissimilar substrates.”

Supreme 62-1 is easy to use and has an advantageous long working life of more than 12 hours after mixing a 100 gram batch. Typical cure schedule options range from 4-6 hours at 140-158°F (60-70°C) to 20-40 minutes at 176-212 °F (80-100°C) to 10-20 minutes at 257°F(125°C). This compound has an elongation of 5-10% and a Shore D hardness of 75-85. Volume resistivity of the cured epoxy exceeds 1014 ohm-cm. Supreme 62-1 is available for purchase in ½ pint, pint, quart, gallon and 5 gallon kits. Specialty packaging in premixed and frozen syringes and in gun kits are available to simplify handling, minimize waste, speed productivity.

Master Bond Thermal Cycling Resistant Adhesives

Master Bond Supreme 62-1 is a two component, high temperature resistant epoxy compound that is able to withstand severe thermal cycling and shock. It offers reliable electrical insulation and protection from a variety of chemicals, including solvents, acids and bases. It has a long working life after mixing and convenient cure schedule options. Read more about Master Bond’s thermal cycling resistant adhesives at https://www.masterbond.com/properties/thermal-cycling-resistant-adhesives or contact Tech Support. Phone: +1-201-343-8983 Fax: +1-201-343-2132 Email: technical@masterbond.com.

Note to Editors:

For a full product description, please visit: https://www.masterbond.com/tds/supreme-62-1

Check out new videos on our YouTube channel: https://www.youtube.com/user/MasterBondVideo

You can embed any of our videos on your website.

CONTACT

James Brenner, Marketing Manager

Email: jbrenner@masterbond.com

Tel: +1-201-343-8983

Fax: +1-201-343-2132

MASTER BOND INC.

154 Hobart Street

Hackensack, NJ 07601-3922

Web: www.masterbond.com

# # #

), glass transition temperature of 155-160°C