This email was sent to (%email%)

Unsubscribe at: http://www.masterbond.com/phplist/?p=unsubscribe&id=2

Subscribe at: http://www.masterbond.com/phplist/?p=subscribe&id=2

----------------------------------

The following is a news release from Master Bond Inc.  You have received it because you are listed as an editor for your publication.

Attached to this email is a low resolution version of the photograph that is included in the press kit for this product.

High resolution versions of this image and files with the body text of this release are available at http://www.masterbond.com/mbnr.html?nr=ep21tdcslo.

----------------------------------

FOR IMMEDIATE RELEASE

Silver Conductive Epoxy Meets 
NASA Low Outgassing Standards

HACKENSACK, NJ – February 8, 2011 – Master Bond EP21TDCS-LO is an electrically conductive silver-filled epoxy that passes ASTM 595 for NASA low outgassing specifications. It is widely used for demanding applications in the electronic, electrical, computer, semiconductor, aerospace and optical industries.

The adhesive cures at room temperature in 24-48 hours and in just 1-2 hrs at 200ºF, producing tough high strength bonds. It features a tensile shear strength of more than 1,800 psi, a T-peel of greater than 5 pli, and outstanding adhesion to similar and dissimilar substrates. The volume resistivity of the system is less than 10-3 ohm-cm. Serviceable over the exceptionally wide range of 4K to 275ºF, EP21TDCS-LO is suitable for cryogenic applications. It withstands thermal cycling and is resistant to chemicals, including water, oil and most organic solvents.

The epoxy offers a simple 1 to 1 mixing ratio by weight or volume. With its low drip formula, EP21TDCS-LO can be conveniently applied with a syringe, knife, spatula or trowel to vertical surfaces without sagging and only contact pressure is required for curing. Syringes have a 3 month shelf life and glass jars have a 6 month shelf life, if stored at room temperature. EP21TDCS-LO is available in premixed and frozen syringes, as well as in metal containers.

Master Bond Electrically Conductive Adhesives

Master Bond’s Polymer System EP21TDCS-LO was developed for high performance bonding and sealing where superior electrical conductivity and NASA low outgassing certification is required. Read more about Master Bond’s electrically conductive adhesives at http://www.masterbond.com/lp/tabs/tp_pp_eleccond.html or contact Technical Support by phone at +1-201-343-8983, by fax at +1-201-343-2132 or by e-mail at technical@masterbond.com.

CONTACT:

James Brenner, Marketing Manager

Email: jbrenner@masterbond.com

Tel: 201-343-8983

Fax: 201-343-2132

MASTER BOND INC.

154 Hobart Street

Hackensack, NJ  07601-3922

Web: www.masterbond.com

# # #